There are two ways to upgrade your router, but before you upgrade it there is something you should know.

- Write down all the current settings as you have to enter them manually after upgrade.

- Choose the firmware which has the same hardware version as yours.

- Do NOT upgrade firmware from a computer with a wireless connection. Use a computer that you can connect to the router with an Ethernet cable. You may damage the router due to the increased packet loss with a wireless connection.

- Do NOT turn off the power during upgrading.

1、If your web-based upgrade page is similar to the follow, please refer to the steps below.

Step 1 Run tftpd32.exe, and keep it on till upgrade successfully.

Step 2 Log into the Router by typing http://192.168.1.1 in the address field of the browser (such as Internet Explorer, Netscape). Both the User Name and Password are admin unless you have changed it.

Step 3 Click System Tools > Firmware, Click Upgrade button, Wait for the router to reboot.

Note: Make the IP address of tftpd32.exe agree with your computer, so does the name of .bin file.

Step 4 Verify that the router’s firmware is upgraded.

Step 5 Reenter the settings you have noted down before upgrading.

2、If your web-based upgrade page is familiar with the follow, please refer to the steps below.

Step 1 Log into the Router by typing http://192.168.1.1 in the address field of the browser (such as Internet Explorer, Netscape). Both the User Name and Password are admin unless you have changed it.

Step 2 Click System Tools > Firmware on the left, click the Browse button to choose the firmware (.bin file) you downloaded before.

Step 3 Click the Upgrade button, Wait for the router to reboot.

Step 4 Verify that the router’s firmware is upgraded.

Step 5 Reenter the settings you have noted down before upgrading.