STANDARD MICROSYSTEMS CORPORATION

SMC SCO LLI 3.4 DRIVER

For 80xx/8216/8416 Ethercard Elite16 Ultra LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide:

o  Lists the software requirements needed to use the SMC Elite16 Ultra SCO

LLI 3.4 Driver with SMC Ethernet (80xx/8216/8416) LAN adapters.

o  Explains how to use the SCO Custom program to install the

SMC 80xx/8216/8416 LLI 3.4 Driver files from the distribution disk.

SOFTWARE REQUIREMENTS

To use the SMC 80xx/8216/8416 SCO LLI 3.4 UNIX Driver files, you must have

networking products that use the drivers. If you are using

SCO UNIX System V/386 Release 3.2 Version 4.2, you need at least

one of the following items:

o  SCO TCP/IP

o  SCO OSI by Retix

o  SCO IPX/SPX

o  Microsoft LAN Manager for UNIX systems, using either SCO TCP

NetBIOS or Microsoft NetBEUI

o  Any compatible stack provided by a third-party vendor

SCO Open Desktop/Open Server Release 3.0 includes TCP/IP and

IPX/SPX, and it can be upgraded to include the rest. If you do

not know which stacks are included with your Open Desktop

system, use "custom(ADM)" to list the products installed on

your system.

INSTALLING the SMC 80xx/8216/8416 SCO LLI 3.4 LAN UNIX DRIVER FILES

If the SMC 80xx/8216/8416 SCO LLI 3.4 Driver files are in DOS format, refer

to the next section, entitled: "CONVERTING THE SMC 80xx/8216/8416 SCO LLI

3.4 Driver FROM DOS FORMAT".

If the SMC 80xx/8216/8416 SCO LLI 3.4 Driver files are in UNIX format, proceed

to "INSTALLING THE SMC 80xx/8216/8416 SCO LLI 3.4 Driver ".

CONVERTING THE SMC 80xx/8216/8416 SCO LLI 3.4 Driver FROM DOS FORMAT

If the SMC 80xx/8216/8416 SCO LLI 3.4 Driver files that you have are in DOS

format, use the following procedure to create a UNIX-compatible

LLI Driver Disk.

Note: If you desire additional configuration information about

your networking stack, refer to the appendixes on the Release

and Installation SCO LLI Driver Disk 3.1.0.

1. Log in as root.

2. Shut down the system and go to System Maintenance mode.

Note: Make sure DOS Merge is running on the system.

3. Insert the DOS version of the SMC 80xx/8216/8416 SCO LLI 3.4 Driver Disk

in an available floppy drive on your computer.

4. Create a temporary directory, then change to it.

5. If you inserted the Standard Microsystems Ethernet LLI Driver

Disk in drive A, type: # doscp a:disk.sme ./disk.sme

If you inserted it in drive B, type: # doscp b:disk.sme ./disk.sme

6. Press the Enter key. The file disk.sme is copied from the

SMC 80xx/8216/8416 SCO LLI 3.4 Driver Disk to your working directory.

7. If you want to verify that the file has been copied to your

working directory, use the "l" command to list the files in your

working directory.

8. The next procedure creates an LLI Driver Disk. It is recommended that

you remove the SMC 80xx/8216/8416 SCO LLI 3.4 Driver Disk from the floppy

drive and insert a blank, tar-formatted 3 1/2-inch disk.  Otherwise, the

files on the SMC 80xx/8216/8416 SCO LLI 3.4 Driver Disk will be overwritten.

9. If you inserted a blank 3 1/2-inch high-density diskette in drive

A, type: # dd if=disk.sme of=/dev/rfd0135ds18 bs=18k

If the disk is in drive B, type:

# dd if=disk.sme of=/dev/rfd1135ds18 bs=18k

10. Press the Enter key. The driver files are copied from your working

directory to the floppy disk.

11. Use the following "tar" command to verify that the files have been

copied:  # tar tvf /dev/rfd0135ds18

You have just created a UNIX-compatible SMC 80xx/8216/8416 SCO LLI Driver Disk.

Continue with steps 3 through 15 under "INSTALLING THE SMC 80xx/8216/8416

SCO LLI Driver", next.

INSTALLING THE SMC 80xx/8216/8416 SCO LLI 3.4 Driver

Use the following procedure to install the SMC SCO LLI LAN UNIX

Driver files.

1. Log in as root.

2. Shut down the system and go to System Maintenance mode by typing

the following command and pressing Enter: shutdown -y -g0 -i1

3. At the shell prompt, type: custom

4. Press the Enter key. The Custom menu appears with the "Install"

option highlighted.

5. Press Enter to begin the installation. The Custom program

highlights "A New Product".

6. Press Enter. The Custom program offers you a choice of installing

the "Entire Product", "Packages", or "Files." The "Entire Product"

option is highlighted.

7. Press Enter to select the "Entire Product" option. The Custom

program prompts you to insert Distribution Floppy in an

available floppy drive.

8. Insert the SMC 80xx/8216/8416 SCO LLI Driver Disk in the floppy drive.

9. Press Enter. After the prep script appears, the Custom program

again prompts you to insert the SMC 80xx/8216/8416 SCO LLI Driver Disk

and highlights the "Continue" option.

10. Since this disk is already in the floppy drive, press Enter to

install the driver. When the driver is installed, the following

message appears at the bottom of your screen.

----------------------------------------------------------------

After installing the SMC 80xx/8216/8416 SCO LLI Driver Disk,

please use netconfig to add drivers to your system. Press

<Enter> to continue.

----------------------------------------------------------------

11. Press any key to continue with the installation. The following

message appears briefly on the screen:

----------------------------------------------------------------

Checking file permissions...

----------------------------------------------------------------

and you return to the main Custom menu. The software list on

your screen now includes the "SMC Elite16 Ultra LLI Driver

Disk".

12. Use the left arrow key to highlight the "Quit" option.

13. Press Enter.

14. Use the appropriate method to configure the adapter.

15. Proceed to the next section to configure the SMC Ethernet driver

for SCO UNIX and the networking stack.

CONFIGURING THE SMC 80xx/8216/8416 SCO LLI 3.4 Driver Disk FOR SCO UNIX

The following procedure describes how to use the "netconfig" utility

to configure the SMC Ethernet driver for SCO UNIX and the networking

stack.

1. Type: netconfig

2. Press the Enter key. The "Available Options: menu" prompt appears.

3. Select "1" to add a chain and press Enter. A list of protocols

appears.

4. Select the protocol you need -- usually "4 sco_tcp", which

corresponds to "SCO TCP/IP for UNIX" in the protocols list.

5. Press Enter. A list of drivers appears, along with the prompt:

"Select next level of chain to Add or q to quit:".

6. Select the number that corresponds to the SMC driver,

"sme0  SMC EtherCard Elite16 Ultra LAN Adapter Driver, board 0",

in the drivers list.

7. Press Enter. The following prompt appears:

"Add chain SCO_tcp --> sme0 (y/n)"

8. Type Y and press Enter. A message tells you that the chain is

being added. After the chain is added, a message prompts you

to select the IRQ setting for the adapter.

9. Select the IRQ setting for the adapter and press Enter.

10. Next, the program prompts you for enter the RAM base address.

Select the RAM base and press Enter.

11. Press Enter. The program prompts you for the Internet

address of this interface.

12. Enter the Internet address for this interface and press Enter.

The program prompts you for the netmask for this interface and

provides a default selection of 255.0.0.0.

13. Press Enter to accept the default, or type a different netmask

for this interface and press Enter. The program asks if the

interface uses a broadcast address of all 1s (default is Y).

14. Answer appropriately. If you respond with Y, enter the

broadcast address for this interface when prompted.

15. The netconfig screen displays your interface address, netmask,

and broadcast address selections, then asks you to verify that

your choices are correct. If your choices are correct, press Y

and Enter. Otherwise, press N and Enter and reenter these

selections.

16. When prompted, enter the local host name chosen for your

system and press Enter. The program asks if you want to

configure the gateway (default is N for no).

17. Press Enter to accept the default and not configure the gateway

at this time. A prompt asks you to select an option or quit

(default is Q for quit).

Note: If you want to configure the gateway when you complete

this procedure, refer to the procedure for doing so in the

appendixes on the Release and Installation SCO LLI Driver

Disk 3.1.0.

18. Press Enter to quit the program and return to the "Available

Options: menu" prompt.

19. Press Q and Enter. The program asks if you want to relink the

kernel.

20. Press Y and Enter to relink the kernel. After the kernel is

relinked, the program asks if you want this kernel to boot by

default.

21. Press Y and Enter to have this kernel boot by default. The

program asks if you want the kernel environment rebuilt.

22. Press Y and Enter to rebuild the kernel environment. After the

kernel is successfully rebuilt, the following message appears:

----------------------------------------------------------------

The kernel has been successfully linked and installed

To activate it, reboot your system

Setting up new kernel environment

----------------------------------------------------------------

The system prompt, #, appears after this message.

23. Run "haltsys" to halt your system, then reboot your system.

Note: Make sure you halt and shut down your system in the proper

manner. Otherwise, you may corrupt system and user files.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.