STANDARD MICROSYSTEMS CORPORATION

SMC 80xx/8216/8416 UNIXWARE 2.x Driver

For SMC Elite16 (Ultra) ISA Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Available drivers.

o  Procedures for preparing a "pkgadd" diskette.

o  Procedures for installing the driver with a "pkgadd" diskette.

AVAILABLE DRIVERS

The Unixware 2.x smeUW2X is designed for the following SMC

Elite16 (Ultra) ISA adapter.

The smeUW2X driver includes:

sme.UW2X - sme driver for Unixware 2.x

PREPARING A "pkgadd" DISKETTE

The following procedures describes how to prepare a "pkgadd" diskette.

In this procedure, variables are denoted by upper-case characters. In

addition, <CR> is used to denote pressing the Enter (or carriage

return) key.

1. Log in as root and enter single user mode.

2. Change to the /tmp subdirectory by entering the following command:

cd /tmp <CR>

3. Insert the DOS diskette that contains DRIVER.UW2X (for example,

sme.UW2X) into the floppy drive. Then use the following doscp

command to copy the DRIVER.UW2X file to the /tmp subdirectory:

doscp X:DRIVER.UW2X DRIVER.UW2X <CR>

In the above command: X is floppy drive A or B.

DRIVER is the driver name, such as sme.

Example: doscp a:sme.UW2X sme.UW2X <CR>

4. The pkgadd format diskette is created by doing as follows:

dd if=DRIVER of=/dev/rfd0135ds18 bs=18k

In the above command: DRIVER is the driver name, such as sme.

Example:

dd if=sme.UW2X of=/dev/rfd0135ds18 bs=18k

INSTALLING THE DRIVER WITH A "pkgadd" DISKETTE

To install the SMC driver using a pkgadd diskette:

1. Type the following command to start the pkgadd process:

pkgadd -d diskette1 <CR>

2. Follow pkgadd's on-screen instructions.

3. Run "/etc/confnet.d/configure -i" and follow the instructions.

4. Restart the computer, UNIXWARE will rebuild the kernal upon boot up

to make the changes effective.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite32 Ultra, Elite Series, EliteView, EZStart,

SuperDisk, and UltraChip are trademarks of Standard Microsystems

Corporation.  Other products and company names are registered

trademarks or trademarks of their respective companies.