STANDARD MICROSYSTEMS CORPORATION

SMC SCO AHS 5.1 NETWORK DRIVERS

For SMC EtherEZ(8416) and EtherCard Elite16 ULTRA(8216), Elite 16(8013) LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide:

o  Lists the software requirements needed to use the SMC EtherEZ and

EtherCard Elite 16 SMCSCO AHS 5.1 Network Drivers v1.00 (sme) with

SMC Ethernet(8416/8216/8013) LAN

adapters.

SOFTWARE REQUIREMENTS

Note: Before installing drivers that are included in the SMCSCO

SuperDisk v.2.0.0a (sme, smpw), be sure to use custom to remove

the existing MDI driver packages (e.g SMCSCO v.1.0.0a/b) on the

target before installing the new v.2.0.0a.

To use the SMC EtherEZ Ethernet SCO AHS 5.1 UNIX

Driver files, you must have networking products that use the drivers.

You must have the following software and hardware systems:

o SCO OpenServer Enterprise Edition, Release 5.0 installed on a

486/Pentium with PCI bus pc machine.

o One or more SMC EtherPower Ethernet Adapters can be installed

in the same machine.

You need at least one of the following items:

o  SCO TCP/IP

o  SCO OSI by Retix

o  SCO IPX/SPX

o  Microsoft LAN Manager for UNIX systems, using either SCO TCP

NetBIOS or Microsoft NetBEUI

o  Any compatible stack provided by a third-party vendor

SCO Open Desktop/Open Server Release 3.0 includes TCP/IP and

IPX/SPX, and it can be upgraded to include the rest. If you do

not know which stacks are included with your Open Desktop

system, use "custom(ADM)" to list the products installed on

your system.

In this release the SCO "custom" program can install the driver from a DOS

diskette. Thus, you need to have the binary distribution file "CUSTOM.DST".

This file must be located under A:\SCO\CUSTOM.DST if you are installing from

the drive A or B:\SCO\CUSTOM.DST if you are installing from drive B.

To create "custom.dst" simply do as follows:

In DOS machine type

1. Create a subdirectory on your floppy disk "\SCO"

2. Copy CUSTOM.DST into \SCO subdirectory that you have created in step 1.

Note: If you desire additional configuration information about

your networking stack, refer to the appendixes on the Release

and Installation SCO OpenServer Network Driver.

INSTALLING THE SMCSCO AHS 5.1 Network Drivers

Use the following procedure to install the SMC SCO MDI LAN UNIX

Driver files.

1. Log in as root.

2. Shut down the system and go to System Maintenance mode by typing

the following command and pressing Enter: shutdown -y -g0 -i1

3. At the shell prompt, type: custom

4. Press the Enter key. The Custom menu appears with the "Software"

option highlighted.

5. Select Software then enter to begin the installation. The Custom program

highlights "Install New".

6. Press Enter. Select the host machine that you want to install from

then press enter.

7. Select Media that you want to install from select Floppy Disk Drive

then press enter.

8. The Custom program offers you a choice of installing

the "Entire Product", "Packages", or "Files." The "Entire Product"

option is highlighted.

9. Press Enter to select the "Entire Product" option. The Custom

program prompts you to insert Distribution Floppy in an

available floppy drive.

10. Insert the SMCSCO AHS 5.1 Network Driver SuperDisk in

the floppy drive. The SMC EtherPower SCO AHS 5.1 Driver binary

distribution disk is available in UNIX or DOS(A or B:\SCO\CUSTOM.DST)

format.

11. Press Enter. After the driver is installed, the following

message appears at the bottom of your screen.

----------------------------------------------------------------

After installing the SMCSCO MDI Drivers SuperDisk,

please use netconfig to add drivers to your system. Press

<Enter> to continue.

----------------------------------------------------------------

12. Press any key to continue with the installation. The following

message appears briefly on the screen:

----------------------------------------------------------------

Checking file permissions...

----------------------------------------------------------------

and you return to the main Custom menu. The software list on

your screen now includes the

"SMCSCO AHS 5.1 Network Drivers (version 2.0.0a)".

13. Use the tab key to highlight the "Hardware" option. Press enter then

select X to exit the custom program.

14. Proceed to the next section to configure the SMC Ethernet driver

for SCO UNIX and the networking stack.

CONFIGURING THE SMC 8416/8216/8013 SCO AHS 5.1 Driver Disk FOR SCO UNIX

The following procedure describes how to use the "netconfig" utility

to configure the SMC EtherEZ(8416), EtherCard Elite ULTRA(8216) and

EtherCard Elite 16 (8013) SCO UNIX and the networking stack.

1. Type: netconfig

2. Press the Enter key. The "Hardware" prompt appears.

3. Presse enter. The sub menu displays the following options:

Add new LAN Adapter

Add new WAN Connection

Remove Network Device

(Modify Hardware Configuration)

(Examine Hardware Conficuration)

Exit

Use up/down arrow key to highlight the "Add new LAN Adapter"

then press enter.

4. In "Add new LAN Adapter" window select "Configure hardware not listed

above...". The complete list of the available drivers are now displayed.

5. Use the arrow key to walk down/up the list. Select the adapter type that

you are going to add it into the system. The following SMC driver and

adapter type are supported in this release.

SMC 8003/8013 Adapter

SMC 8416/8216 Adapter

Select Adapter type that you are going to configure.

6. The "Search for network adapter" is now appeared and the question

"Do you wish to search for a SMC XXXX/XXXX Adapter ?"

is hightlighted.

7. The net card detection program will detect if the selected adapter

in the system and hightlight its configuration (IO Address,

Ram Address, Interrupt level (IRQ)....

If you agreed with the detected configuration then go to step 8

in the next section to select network protocol.

8. You are here to make change to the configuration of the card.

9. Link the desire protocol stack to the SMC EtherPower driver.

10. Select Hardware->Exit to quit from netconfig, you will be prompted to

relink the kernel.

Answer OK to relink the kernel and rebuild the kernel environment.

11. After the kernel is successfully rebuilt, the following message appears:

----------------------------------------------------------------

The kernel has been successfully linked and installed

To activate it, reboot your system

Setting up new kernel environment

----------------------------------------------------------------

The system prompt, #, appears after this message.

12. Run "haltsys" to halt your system, then reboot your system.

Note: Make sure you halt and shut down your system in the proper

manner. Otherwise, you may corrupt system and user files.

SMC and Standard Microsystems are registered trademarks; and

EtherPower Ethernet and Fast Ethernet,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.

#

#

#