DEC 1994

NDIS Driver Program Installation

==================================

The NDIS driver programs in this disk support the following UMC cards:

UM9008

This diskette is labeled as LAN_DRV_UMC and the files are organized as follows:

Directory              Contents

----------------       ----------------------------------------------

\NDIS\DOS        NDIS driver for DOS.

UMC NDIS driver program for DOS follows the Microsoft's NDIS spec v2.0.1.

UMC NDIS driver program support the following network operating systems:

MS LAN Manager v2.x

DECnet PathWork v4.x

WIN/TCP PathWay Access for DOS v1.x

3COM 3+ Open v1.1

Banyan VINES v4.00(2)

IBM LAN Support Program v1.3.x

Windows for Workgroups v3.1x

IBM OS/2 2.0 / LAN Server v2.0

IBM LAN Server v3.0

IBM Communication Manager v1.0

Artisoft LANtastic v6.0

***********************************************

*  How to install UM9008.DOS on MS LAN MANAGER *

***********************************************

1)  Prepare a diskette with volume name "DOS_DRV_UMC" and create directories

in the following manner:

>label a: DOS_DRV_DLK     -- Insert a blank disk in drive 'A'.

>md MSLANMAN.DOS          -- Make a directory called MSLANMAN.DOS.

>cd MSLANMAN.DOS

>md DRIVERS               -- Make a directory called DRIVERS.

>cd DRIVERS

>md ETHERNET              -- Make a directory called ETHERNET.

>md NIF                   -- Make a directory called NIF.

>cd ETHERNET

>md UM9008                -- Make a directory called UM9008.

MSLANMAN.DOS  (root)

|

DRIVERS

________|_________

|                  |

ETHERNET             NIF

_____|______

|            |

UM9008

2) Insert the UM9008 driver program diskette in drive 'A' and insert the

prepared diskette (DOS_DRV_UMC) in drive 'B'.  Afterwards, copy the

appropriate NDIS driver files from drive 'A' to 'B'.

>a:                     -- Go to drive 'A'.

>cd NDIS\DOS

>copy . B:\MSLANMAN.DOS\DRIVERS\ETHERNET\*.*

>copy UM9008.DOS     B:\MSLANMAN.DOS\DRIVERS\ETHERNET\UM9008

>copy PROTOCOL.INI   B:\MSLANMAN.DOS\DRIVERS\ETHERNET\UM9008

>copy UM9008.NIF     B:\MSLANMAN.DOS\DRIVERS\NIF

3) MS LAN Manager provides a SETUP.EXE program on their Setup disk.

Execute this SETUP program to specify the proper driver for UM9008 card.

-If you are installing LAN Manager for the first time, do the following:

Upon completion of retrieval of the last workstation's disk contents, you

will be asked if you want to import additional network drivers.

Answer "YES".

-If you have already installed LAN Manager in your harddisk, do the following:

Select the Configuration menu, and then select "Add New Config" option.

Afterwards, choose the "Other Driver" option.

4) Insert the prepared diskette (DOS_DRV_UMC) in drive 'A' and follow the

instructions on the screen.  The SETUP program will copy the necessary

NDIS driver programs from the DOS_DRV_UMC diskette to your hardisk (into

the LAN Manager directory).

5) After completion of the SETUP, your CONFIG.SYS file will be modified and

contain statements referring to the UMC NDIS driver programs.

The statements added in CONFIG.SYS are as follows:

DEVICE=C:\LANMAN\DRIVERS\PROTMAN\PORTMAN.DOS /I:C:\LANMAN

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\UM9008\UM9008.DOS

Reboot your computer and follow the NDIS binding procedure to use the UMC

NDIS driver programs.

*******************************************************

*  How to install UM9008.DOS on WIN/TCP PathWay Access *

*******************************************************

1) Copy UM9008.DOS to directory C:\LANMAN.

2) Go to the directory C:\LANMAN, and create a text file called PROTOCOL.INI:

[UM9008]

drivername = UM9008$

3) Add the following statements to your CONFIG.SYS file:

device=c:\lanman\UM9008.DOS

4) Add the following statements to your AUTOEXEC.BAT file:

ndis -d:1

winbind

wintcp

************************************************

* How to install UM9008.DOS on DECnet PathWork *

************************************************

You can use the DECnet PathWork's  NETSET.EXE program to install UMC NDIS

driver program.  Please refer to  DECnet  installation manual for detailed

information.    The following instructions assume that you have already

installed the DECnet PathWork with 3Com's Etherlink II card.

1) Modify your CONFIG.SYS file as follows:

device = c:\decnet\ELNKII.SYS

to

device=c:\decnet\UM9008.DOS

2) Modify the PROTOCOL.INI file as follows:

from:

---------------------------------

[ELNKMACII]

drivername = ELNKII$

ioaddress = 0x300

interrupt = 3

xmitbufs = 2

[IPX4MAC]

:

bindings = ELNKMACII

:

[DATALINK]

:

bindings = ELNKMACII

:

to:

-----------------------------------

[UM9008]

drivername = UM9008$

[IPX4MAC]

:

bindings = UM9008

:

[DATALINK]

:

bindings = UM9008

:

********************************************

* How to install UM9008.DOS on 3Com 3+ Open *

********************************************

Supposing that you have installed the 3Com 3+ Open with 3Com's 3C501 card:

1) Copy UM9008.DOS to  C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

2) Modify the device driver statement for 3C501 card in your CONFIG.SYS to:

device=c:\3open\doswksta\lanman\drivers\UM9008.DOS

3) Modify the PROTOCOL.INI file to:

[ETHERLINK]

:

drivername = UM9008$

:

********************************************

* How to install UM9008.DOS on Banyan VINES *

********************************************

1. Create a bootable diskette (booted with DOS 3.1 or above) which will be

used as boot diskette of Banyan VINES workstation or create a sub-directory

on your hard disk.

NOTE : To install UMC NDIS driver program, you need BANYAN VINES v4.00(2)

or above.

There are two methods to copy the required files for Banyan VINES workstation

to the bootable diskette or sub-directory of hard disk.

Method 1.

1) Execute 'INSTALL' command which was saved in the diskette labeled

"VINES MASTER DISK".

2) Follow the instructions to copy the files to the bootable diskette

or sub-directory of hard disk.

Method 2.

1) Execute 'BAN' command.

2) Login VINES system.

3) Execute 'PCCOPY' command.

4) Select 'NDIS Ethernet' item and follow instructions as menu driven

for copying the files to the bootable diskette or sub-directory of

hard disk.

2. Insert the UM9008 driver program diskette in drive 'A' and insert the

prepared bootable diskette in drive 'B'. Afterwards, copy the appropriate

NDIS driver files UM9008.DOS from drive 'A' to 'B' or sub-directory of hard

disk.

3. Create a CONFIG.SYS file which contains the following statements:

device=protman.dos /i:c:\

device= [driver]: \path\UM9008.DOS

NOTE: " A:\" is the path which may be changed depending on the location of

the files for BANYAN VINES workstation.

4. Create a UM9008.NIF file and include the following statements in this file:

Model = UMC UM9008 Ethernet Card

Path = A:\

DeviceDriver = UM9008.DOS

DeviceName = UM9008$

Type = NDIS

NOTE:" A:\" is the path which may be changed dependeding on the location

of the files for BANYAN VINES workstation.

5. Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "UM9008"

[UM9008]

drivername = UM9008$

6. Execute 'PCCONFIG' command

1) When "CONFIGURING A PC FOR THE NETWORK" is displayed on the screen

then select item '1 - Network Card Settings'.

|

When "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

When "NDIS Ethernet" is displayed on the screen

then set your choice for UMC NIC

Ex. Interrupt Level (2-15) : 3

PROTOCOL.INI bindings = UM9008

|

2) Return to Screen "CONFIGURING A PC FOR THE NETWORK"

then select item '2 - Login Environment Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

Then 'PCCONFIG.DB' file will be generated.

7. Reboot your computer.

8. Login into VINES by executing the 'BAN' command.

*******************************************************

* How to install UM9008.DOS on IBM LAN Support Program *

*******************************************************

Supposing that you have installed the IBM LAN Support Program in your computer,

you can use UM9008.DOS to replace the installed driver program.

Follow the instructions below to change the contents of CONFIG.SYS and

PROTOCOL.INI files.

1. Copy UM9008.DOS to the directory where your IBM LAN Support program

resides.

2. Modify the original NDIS device driver statement in CONFIG.SYS as

follows:

device= [driver]:\path\UM9008.DOS

3. Modify the PROTOCOL.INI file as follows:

BINDINGS = UM9008

:

:

[UM9008]

DRIVERNAME = UM9008$

******************************************************

* How to install UM9008.DOS on Windows for Workgroups *

******************************************************

1. Manual editing of pre-selected configuration.  Not recommended if you

are using Windows for Workgroups 3.11.

Involves manual editing of installed Windows for Workgroups configuration

files using pre-selected NE2000 driver.

Please refer to WFW1.DOC for detailed installation procedures.

2. Menu-driven installation.  (Recommended method of installation)

Using a UMC supplied OEMSETUP.INF file, the user can choose to install

the appropriate UMC adapters through a menu.

Please refer to WFW2.DOC for detailed installation procedures.

************************************************************

* How to install UM9008.DOS NDIS Driver on LANtastic v 6.0  *

************************************************************

1. Install LANtastic 6.0 as instructed by Artisoft.  When you are asked to

select a network driver, choose "NDIS SUPPORT FOR NETWORK ADAPTERS".

Afterwards, insert the UMC Driver disk in drive 'B' and type its path

when asked for the NDIS Driver Directory:  B:\NDIS2\DOS

2. When you finish the installation, review the following files to make sure

that it contains the appropriate commands:

\CONFIG.SYS

===========

:

Device=c:\lantasti\protman.dos /i:c:\lantasti

Device=c:\lantasti\UM9008.DOS

\AUTOEXEC.BAT

=============

:

call c:\lantasti\startnet.bat

\LANTASTI\STARTNET.BAT

======================

:

Loadhigh AI-NDIS Bind_To=UM9008_NIF

AILANBIO @Startnet.cfg

:

\LANTASTI\PROTOCOL.INI

======================

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

[UM9008_NIF]

Drivername = UM9008$

********

* NOTE *

********

==============================================================================

UMC NDIS PROTOCOL.INI User Guide

The keywords used to create the PROTOCOL.INI file for UM9008 is as follows:

DRIVERNAME - This keyword specifies the NIC driver's name.

* Following is an example PROTOCOL.INI for UMC Ethernet product:

Drivername = UM9008$

==============================================================================

UMC NDIS PROTOCOL.INI User Guide

The keywords used to create the PROTOCOL.INI file for UM9008 is as follow:

DRIVERNAME - This specifies the network card's driver name.

Following is an example of a PROTOCOL.INI file that can be used with UM9008

Ethernet Card:

[UM9008]

DriverName = UM9008$